Quantum control and process tomography of a semiconductor quantum dot hybrid quibit

Quantum dots in semiconductors are promising hosts for quantum bits: small numbers of electrons can be trapped and manipulated, and at low temperatures it is possible to manipulate such electrons faster than they decohere.  Recently it was proposed theoretically that a fast and high fidelity qubit could be formed from three electrons in a semiconductor double quantum dot.  In this talk I discuss new results demonstrating the manipulation and measurement of such a qubit [1].  Initialization of the qubit is performed by tuning the gate voltages of a Si/SiGe double quantum dot so that the charge occupation is (2,1) and then applying a sequence of carefully controlled gate voltage pulses.  Measurement is performed by mapping qubit logical 0 to the (2,1) charge state and qubit logical 1 to the (1,2) charge state.  We show that the state of the qubit can be manipulated to arbitrary points on the Bloch sphere, and we demonstrate both X and Z rotations.  Full process tomography of these gates is performed, and we extract fidelities of at least 85% for X gates and 94% for Z gates.  This work was supported in part by ARO (W911NF-12-0607).  Development and maintenance of the growth facilities used for fabricating samples is supported by DOE (DE-FG02-03ER46028).

[1] D. Kim, Z. Shi, C. B. Simmons, D. R. Ward, J. R. Prance, Teck Seng Koh, John King Gamble, D. E. Savage, M. G. Lagally, Mark Friesen, S. N. Coppersmith, and M. A. Eriksson, Nature 511, 70 (2014).

Department of Physics and Astronomy, Purdue University

# Condensed Matter Seminar

Friday, November 14, 2014

3:30 PM, Room 203 Physics

(Refreshments 3:00 PM, Room 242)

Professor Mark Eriksson

College of Engineering, Materials Science Program

University of Wisconsin-Madison