Phase Change Material Integrated Silicon Photonics: GST and Beyond

Arka Majumdar, Associate Professor

Electrical and Computer Engineering and Physics

University of Washington, Seattle

Reconfiguration of silicon photonic integrated circuits relying on the weak, volatile thermo-optic or electro-optic effect of silicon usually suffers from large footprint and energy consumption. Non-volatile phase change materials, such as Ge2Sb2Te5 (GST) provide an attractive solution to solve this problem. I will talk about our effort on integrating GST with silicon photonics to demonstrate an energy-efficient, compact, non-volatile, reprogrammable platform. By adjusting the energy and number of free-space laser pulses applied to the GST, we characterized the strong broadband attenuation and optical phase modulation effects of the platform, and perform quasi-continuous tuning enabled by the thermo-optically induced phase changes. As a result, a non-volatile optical switch with high extinction ratio as large as 33 dB is demonstrated [1]. We also solved the problem of large absorptive loss in GST by designing a new type of directional coupler, and demonstrated an optical switch with less than 1dB insertion loss, even though the material itself has very high loss [2]. Using silicon p-i-n heater, we demonstrated electrical actuation of the phase transition to create a prototype of electrically controlled reconfigurable switch with ultra-low power consumption [3,4]. Finally, I will discuss new phase change materials, and device engineering to create next generation low-loss reconfigurable optical switch and phase-shifters.

Bio: Prof. Arka Majumdar is an Associate Professor in the departments of Electrical and Computer Engineering and Physics at the University of Washington, Seattle. He received his B. Tech. degree from the Indian Institute of Technology, Kharagpur in 2007, where he was honored with the President’s Gold Medal. Majumdar completed his master’s degree (2009) and Ph.D. (2012) in Electrical Engineering at Stanford University. He spent one year at the University of California, Berkeley (2012-13) and also at Intel Labs as a postdoc in Santa Clara, CA (2013-14). His research interests include developing a hybrid integrated nanophotonic platform using emerging material systems for applications in ultra-low power optical information science, imaging, and microscopy. Majumdar is the recipient of the Young Investigator Award from the Air Force Office of Scientific Research (2015), Intel Early Career Faculty Award (2015), Alfred P. Sloan Research Fellowship for Physics (2018), NSF Career Award (2019), the UW College of Engineering Outstanding Young Faculty Award (2019), and Office of Naval Research Young Investigator Award (2020). He also cofounded Tunoptix, a startup developing mechanically tunable metasurface technology for machine vision and augmented reality visors.

References:

[1]	J. Zheng, A. Khanolkar, P. Xu, S. Colburn, S. Deshmukh, J. Myers, J. Frantz, E. Pop, J. Hendrickson, J. Doylend, N. Boechler, and A. Majumdar, "GST-on-silicon hybrid nanophotonic integrated circuits: a non-volatile quasi-continuously reprogrammable platform," Optical Materials Express, vol. 8, pp. 1551-1561, 2018/06/01 2018.

[2]	P. Xu, J. Zheng, J. K. Doylend, and A. Majumdar, "Low-Loss and Broadband Nonvolatile Phase-Change Directional Coupler Switches," ACS Photonics, vol. 6, pp. 553-557, 2019/02/20 2019.

[3]	J. Zheng, Z. Fang, C. Wu, S. Zhu, P. Xu, J. K. Doylend, S. Deshmukh, E. Pop, S. Dunham, M. Li, and A. Majumdar, "Nonvolatile Electrically Reconfigurable Integrated Photonic Switch Enabled by a Silicon PIN Diode Heater," Advanced Materials, vol. 32, p. 2001218, 2020/08/01 2020.

[4]	J. Zheng, S. Zhu, P. Xu, S. Dunham, and A. Majumdar, "Modeling Electrical Switching of Nonvolatile Phase-Change Integrated Nanophotonic Structures with Graphene Heaters," ACS Applied Materials & Interfaces, vol. 12, pp. 21827-21836, 2020/05/13 2020.